ARROW 3 WEAPON SYSTEM

Environmental and RAM Requirements

Climatic, dynamic, EMC and reliability requirements for all AWS-3 segments

| Document number | A3-ENV-050 |
| --- | --- |
| Revision | B |
| Date | 30 January 2026 |
| Status | Baselined |
| Prepared by | Specialty Engineering |
| Classification | UNCLASSIFIED // FICTITIOUS DEMONSTRATION DATA |

Revision history

| Rev | Date | Description | CCB ref. |
| --- | --- | --- | --- |
| A | 22 May 2025 | Initial release | CCB-25-015 |
| B | 30 Jan 2026 | Low operating temperature extended to −33 °C; interceptor MTBF raised to 2,000 h | CCB-26-001 |

Approvals

| Role | Organisation | Signature | Date |
| --- | --- | --- | --- |
| Specialty Engineering lead | Weapon System Engineering | e-signed | 30 Jan 2026 |
| Chief Systems Engineer | Weapon System Engineering | e-signed | 02 Feb 2026 |

# 1. Scope

This document defines the natural and induced environments, EMC requirements and reliability allocations that apply to every field-deployed item of AWS-3.

# 2. Applicable documents

| Document | Title | Rev |
| --- | --- | --- |
| A3-SRS-001 | System Requirements Specification | C |
| MIL-STD-810H | Environmental Engineering Considerations and Laboratory Tests | H |
| MIL-STD-461G | Control of Electromagnetic Interference Characteristics | G |

# 3. Environmental requirements

Verification methods: A = Analysis, I = Inspection, D = Demonstration, T = Test.

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| ENV-TMP-001 | All field-deployed equipment, including the all-up round in its canister, shall operate at ambient temperatures from −33 °C to +55 °C. | T | SRS-ENV-002 |
| ENV-TMP-002 | Non-operating (storage and transport) temperature range shall be −40 °C to +71 °C. | T | SRS-ENV-001 |
| ENV-HUM-003 | Equipment shall withstand humidity per MIL-STD-810H Method 507.6, Procedure II. | T | SRS-ENV-001 |
| ENV-SND-004 | Equipment shall withstand blowing sand and dust per MIL-STD-810H Method 510.7. | T | SRS-ENV-001 |
| ENV-VIB-005 | Transport vibration shall be per MIL-STD-810H Method 514.8, Category 4 (truck transportation). | T | SRS-ENV-001 |
| ENV-EMC-010 | All segments shall meet MIL-STD-461G requirements CE102, CS101, CS114, RE102 and RS103 as tailored in Annex B. | T | SRS-EMC-001 |

# 4. Reliability allocations

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| RAM-004 | Interceptor (all-up round) captive-carry MTBF, powered in the canister on the launcher, shall be at least 2,000 h. | A | SRS-RAM-004 |
| RAM-005 | Launcher MTBF shall be at least 1,200 h. | A | SRS-RAM-004 |
| RAM-006 | FCR MTBCF shall be at least 500 h. | A | SRS-RAM-004 |
| RAM-010 | Interceptor storage life in the sealed canister shall be at least 15 years. | A | SRS-RAM-010 |

| Acronym | Meaning |
| --- | --- |
| AUR | All-up round (interceptor in its canister) |
| BMC | Battle Management Center |
| CCB | Configuration Control Board |
| ECEF | Earth-Centred, Earth-Fixed |
| FCR | Fire Control Radar |
| ICD | Interface Control Document |
| IFTU | In-flight target update |
| LCU | Launcher Control Unit |
| MTBF | Mean time between failures |
| PTL | Prepare-to-Launch command |
| RT | Remote terminal (MIL-STD-1553B) |
| TBR | To be reviewed |