ARROW 3 WEAPON SYSTEM

Fire Control Radar Segment Specification

Surveillance, tracking and fire-control radar for AWS-3

| Document number | A3-FCR-SSS-020 |
| --- | --- |
| Revision | B |
| Date | 09 April 2026 |
| Status | Baselined |
| Prepared by | Sensors Integrated Product Team |
| Classification | UNCLASSIFIED // FICTITIOUS DEMONSTRATION DATA |

Revision history

| Rev | Date | Description | CCB ref. |
| --- | --- | --- | --- |
| A | 15 Aug 2025 | Initial release | CCB-25-027 |
| B | 09 Apr 2026 | Track message rate and time-tag accuracy aligned with SRS Rev C draft | CCB-26-008 |

Approvals

| Role | Organisation | Signature | Date |
| --- | --- | --- | --- |
| Sensors Segment lead | Sensors IPT | e-signed | 09 Apr 2026 |
| Chief Systems Engineer | Weapon System Engineering | e-signed | 10 Apr 2026 |

# 1. Scope

This specification defines the requirements of the Fire Control Radar (FCR) Segment of AWS-3.

Detection and tracking performance is specified in the classified annex A3-SRS-001-CA.

# 2. Applicable documents

| Document | Title | Rev |
| --- | --- | --- |
| A3-SRS-001 | System Requirements Specification | C |
| A3-ICD-120 | BMC–FCR–Launcher Data Interface Control Document | C |
| A3-ENV-050 | Environmental and RAM Requirements | B |

# 3. Requirements

Verification methods: A = Analysis, I = Inspection, D = Demonstration, T = Test.

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| FCR-TRK-010 | The FCR shall maintain tracks on all threats within its coverage as specified in A3-SRS-001-CA. | T | SRS-TL-003 |
| FCR-TRK-012 | The FCR shall report target track state vectors to the BMC at not less than 10 Hz per engaged track, in WGS-84 ECEF coordinates. | T | SRS-NAV-002 |
| FCR-TIM-003 | Track messages shall carry a time tag accurate to ±1 µs of system time. | T | SRS-TIM-001 |
| FCR-IF-001 | All FCR–BMC data exchange shall conform to A3-ICD-120 Rev C. | I | SRS-IF-002 |
| FCR-ENV-001 | The FCR shall meet A3-ENV-050 Rev B for field-deployed equipment. | T | SRS-ENV-001 |
| FCR-EMC-002 | The FCR shall meet MIL-STD-461G, tailored per A3-ENV-050 Rev B. | T | SRS-EMC-001 |
| FCR-RAM-004 | The FCR mean time between critical failures (MTBCF) shall be at least 500 h. | A | SRS-RAM-004 |

| Acronym | Meaning |
| --- | --- |
| AUR | All-up round (interceptor in its canister) |
| BMC | Battle Management Center |
| CCB | Configuration Control Board |
| ECEF | Earth-Centred, Earth-Fixed |
| FCR | Fire Control Radar |
| ICD | Interface Control Document |
| IFTU | In-flight target update |
| LCU | Launcher Control Unit |
| MTBF | Mean time between failures |
| PTL | Prepare-to-Launch command |
| RT | Remote terminal (MIL-STD-1553B) |
| TBR | To be reviewed |