ARROW 3 WEAPON SYSTEM

Interceptor–Launcher Interface Control Document

Mechanical, electrical, data and sequencing interface between the all-up round and the launcher

| Document number | A3-ICD-110 |
| --- | --- |
| Revision | C |
| Date | 03 July 2026 |
| Status | Baselined |
| Prepared by | Interface Working Group (INT / LCH) |
| Classification | UNCLASSIFIED // FICTITIOUS DEMONSTRATION DATA |

Revision history

| Rev | Date | Description | CCB ref. |
| --- | --- | --- | --- |
| A | 11 Apr 2025 | Initial release | CCB-25-012 |
| B | 07 Nov 2025 | Data bus RT addressing and message set Table 4 added | CCB-25-038 |
| C | 03 Jul 2026 | Pre-launch current limit reduced to 12 A; READY timing aligned with SRS-TL-002 | CCB-26-022 |

Approvals

| Role | Organisation | Signature | Date |
| --- | --- | --- | --- |
| Interface Working Group chair | Weapon System Engineering | e-signed | 03 Jul 2026 |
| Interceptor Segment lead | Interceptor IPT | e-signed | 04 Jul 2026 |
| Launcher Segment lead | Launcher IPT | e-signed | 04 Jul 2026 |

# 1. Scope

This ICD defines the interface between the Arrow 3 all-up round (AUR: interceptor in its canister) and the launcher canister cell. Both the Interceptor Segment and the Launcher Segment shall comply with it.

# 2. Applicable documents

| Document | Title | Rev |
| --- | --- | --- |
| A3-SRS-001 | System Requirements Specification | C |
| A3-LCH-SSS-040 | Launcher Segment Specification | B |
| A3-LCH-DWG-2210 | Canister envelope and interface drawing | D |
| MIL-STD-1553B | Digital Time Division Command/Response Multiplex Data Bus (Notice 4) | B |
| MIL-DTL-38999 | Connectors, Electrical, Circular, Series III | L |
| MIL-STD-1275E | 28 VDC Electrical Systems in Military Vehicles | E |

# 3. Interface requirements

Verification methods: A = Analysis, I = Inspection, D = Demonstration, T = Test.

## 3.1 Mechanical

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| IF-MEC-001 | The canister outer envelope and attachment points shall conform to drawing A3-LCH-DWG-2210 Rev D. | I | SRS-IF-001 |
| IF-MEC-002 | The mass of the loaded canister (all-up round) shall not exceed 1,350 kg. | I | SRS-IF-001 |
| IF-MEC-003 | The centre of gravity of the AUR shall lie within the envelope of A3-LCH-DWG-2210 Rev D, sheet 4. | A | IF-MEC-001 |

## 3.2 Electrical

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| IF-PWR-001 | The launcher shall supply 28 VDC (MIL-STD-1275E) to each canister cell through umbilical connector J1. | T | SRS-PWR-002 |
| IF-PWR-003 | The current drawn by one AUR from the launcher supply shall not exceed 12 A continuous in any pre-launch state, including IMU run-up and thermal conditioning. | T | SRS-PWR-002 |
| IF-PWR-004 | Inrush current at power application shall not exceed 20 A for more than 50 ms. | T | IF-PWR-003 |
| IF-CON-005 | Umbilical connector J1 shall be MIL-DTL-38999 Series III, shell size 25, 61 contacts; pin allocation per Table 3. | I | — |

## 3.3 Data

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| IF-DAT-001 | Data between the Launcher Control Unit and the AUR shall be exchanged over a dual-redundant MIL-STD-1553B bus (Bus A / Bus B). The LCU shall be bus controller; the interceptor shall be remote terminal RT 05. | T | SRS-IF-001 |
| IF-DAT-004 | The message set shall be as defined in Table 4: PTL (Prepare-to-Launch), STS (status), ALN (transfer alignment), TGT (pre-launch target data), LCH (launch) and ABT (abort). | I | IF-DAT-001 |
| IF-NAV-006 | Pre-launch target data (message TGT) shall be expressed in WGS-84 ECEF coordinates per SRS-NAV-002. | I | SRS-NAV-002 |

## 3.4 Sequencing and timing

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| IF-SEQ-006 | The interceptor shall report READY (message STS, bit 0) within 3.0 s of receiving PTL. | T | SRS-TL-002 |
| IF-SEQ-007 | The launcher shall not issue LCH before READY. If READY is not received within 3.5 s of PTL, the LCU shall declare HANG-FIRE and select the next canister. | T | SRS-SAF-002 |
| IF-TIM-010 | The LCU shall distribute system time to the AUR by 1553B time-tag message plus a 1 PPS discrete; the interceptor shall align its time reference within ±1 µs. | T | SRS-TIM-001 |

# 4. Notes

Table 3 (pin allocation) and Table 4 (message formats) are maintained in the interface database IFDB-A3, export of 01 July 2026.

| Acronym | Meaning |
| --- | --- |
| AUR | All-up round (interceptor in its canister) |
| BMC | Battle Management Center |
| CCB | Configuration Control Board |
| ECEF | Earth-Centred, Earth-Fixed |
| FCR | Fire Control Radar |
| ICD | Interface Control Document |
| IFTU | In-flight target update |
| LCU | Launcher Control Unit |
| MTBF | Mean time between failures |
| PTL | Prepare-to-Launch command |
| RT | Remote terminal (MIL-STD-1553B) |
| TBR | To be reviewed |