ARROW 3 WEAPON SYSTEM

Launcher Segment Specification

Road-mobile launcher with six canister cells and Launcher Control Unit

| Document number | A3-LCH-SSS-040 |
| --- | --- |
| Revision | B |
| Date | 21 May 2026 |
| Status | Baselined |
| Prepared by | Launcher Integrated Product Team |
| Classification | UNCLASSIFIED // FICTITIOUS DEMONSTRATION DATA |

Revision history

| Rev | Date | Description | CCB ref. |
| --- | --- | --- | --- |
| A | 30 Jun 2025 | Initial release | CCB-25-019 |
| B | 21 May 2026 | Power distribution and erector rating updated for AUR mass limit | CCB-26-011 |

Approvals

| Role | Organisation | Signature | Date |
| --- | --- | --- | --- |
| Launcher Segment lead | Launcher IPT | e-signed | 21 May 2026 |
| Chief Systems Engineer | Weapon System Engineering | e-signed | 22 May 2026 |

# 1. Scope

This specification defines the requirements of the Arrow 3 Launcher Segment: the launcher vehicle, erector, six canister cells, power distribution unit (PDU) and Launcher Control Unit (LCU).

# 2. Applicable documents

| Document | Title | Rev |
| --- | --- | --- |
| A3-SRS-001 | System Requirements Specification | C |
| A3-ICD-110 | Interceptor–Launcher Interface Control Document | C |
| A3-ICD-120 | BMC–FCR–Launcher Data Interface Control Document | C |
| A3-ENV-050 | Environmental and RAM Requirements | B |

# 3. Requirements

Verification methods: A = Analysis, I = Inspection, D = Demonstration, T = Test.

## 3.1 Mechanical and mobility

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| LCH-MOB-001 | The launcher shall be transportable on public roads without special permits when stowed. | D | Logistics concept |
| LCH-MEC-014 | The erector shall be rated for loaded canisters (all-up rounds) of up to 1,350 kg per cell, with a structural safety factor of 1.5. | A | IF-MEC-002 |
| LCH-MEC-016 | The erector shall raise the canister pack from stowed to launch elevation in not more than 60 s. | T | — |

## 3.2 Power

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| LCH-PWR-020 | The launcher shall be capable of autonomous operation from its own generator for at least 24 h. | D | — |
| LCH-PWR-021 | The PDU shall supply 28 VDC (MIL-STD-1275E) to each of the six canister cells, limited to 12 A continuous per cell by electronic circuit breakers. | T | IF-PWR-003 |

## 3.3 Control and data

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| LCH-DAT-010 | The LCU shall act as MIL-STD-1553B bus controller on one dual-redundant bus per canister cell. | T | IF-DAT-001 |
| LCH-DAT-012 | The LCU shall exchange engagement commands and status with the BMC per A3-ICD-120 Rev C. | T | SRS-IF-002 |
| LCH-TL-003 | The LCU shall issue PTL to the selected canister within 3.0 s of receiving a valid engagement command, and shall issue LCH within 50 ms of receiving READY. | T | SRS-TL-002 |
| LCH-TIM-005 | The LCU shall distribute system time to each canister cell within ±1 µs of the BMC master clock. | T | SRS-TIM-001 |

## 3.4 Environment

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| LCH-ENV-030 | The launcher shall operate at ambient temperatures from −33 °C to +55 °C. | T | SRS-ENV-002 |
| LCH-ENV-031 | The launcher shall meet the transport shock and vibration requirements of A3-ENV-050 Rev B. | T | SRS-ENV-001 |

## 3.5 Safety

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| LCH-SAF-040 | The launcher shall provide the launcher-controlled ignition inhibit required by SRS-SAF-002. | D | SRS-SAF-002 |

| Acronym | Meaning |
| --- | --- |
| AUR | All-up round (interceptor in its canister) |
| BMC | Battle Management Center |
| CCB | Configuration Control Board |
| ECEF | Earth-Centred, Earth-Fixed |
| FCR | Fire Control Radar |
| ICD | Interface Control Document |
| IFTU | In-flight target update |
| LCU | Launcher Control Unit |
| MTBF | Mean time between failures |
| PTL | Prepare-to-Launch command |
| RT | Remote terminal (MIL-STD-1553B) |
| TBR | To be reviewed |