ARROW 3 WEAPON SYSTEM

System Requirements Specification

Arrow 3 Weapon System (AWS-3) — top-level requirements

| Document number | A3-SRS-001 |
| --- | --- |
| Revision | C |
| Date | 14 June 2026 |
| Status | Baselined (SRR action items closed) |
| Prepared by | Weapon System Engineering Directorate |
| Classification | UNCLASSIFIED // FICTITIOUS DEMONSTRATION DATA |

Revision history

| Rev | Date | Description | CCB ref. |
| --- | --- | --- | --- |
| A | 02 Feb 2025 | Initial release for System Requirements Review | CCB-25-004 |
| B | 19 Sep 2025 | Timeline allocation added (SRS-TL-002); RAM figures updated | CCB-25-031 |
| C | 14 Jun 2026 | Time-sync and reference-frame requirements made mandatory for all segments | CCB-26-017 |

Approvals

| Role | Organisation | Signature | Date |
| --- | --- | --- | --- |
| Chief Systems Engineer | Weapon System Engineering | e-signed | 14 Jun 2026 |
| Program Manager | Program Office | e-signed | 15 Jun 2026 |
| Quality Assurance | Product Assurance | e-signed | 15 Jun 2026 |

# 1. Scope

This System Requirements Specification (SRS) establishes the functional, interface, environmental, reliability and safety requirements of the Arrow 3 Weapon System (AWS-3), an exo-atmospheric ballistic missile defence system. It is the parent document of all segment specifications and interface control documents listed in Section 2.

Kill-chain performance values (detection, engagement envelope and probability figures) are contained in the classified annex A3-SRS-001-CA and are not reproduced in this document.

## 1.1 System overview

AWS-3 comprises five segments, each specified in its own segment specification:

Interceptor Segment (INT) — two-stage interceptor with an exo-atmospheric kill vehicle, delivered as an all-up round in a sealed canister.

Launcher Segment (LCH) — road-mobile launchers, six canister cells each, with a Launcher Control Unit (LCU).

Fire Control Radar Segment (FCR) — long-range surveillance and fire-control radar.

Battle Management Center (BMC) — engagement planning, weapon assignment and in-flight target updates.

Communications Segment (COM) — tactical data network linking all segments.

# 2. Applicable documents

| Document | Title | Rev |
| --- | --- | --- |
| A3-ICD-110 | Interceptor–Launcher Interface Control Document | C |
| A3-ICD-120 | BMC–FCR–Launcher Data Interface Control Document | C |
| A3-LCH-SSS-040 | Launcher Segment Specification | B |
| A3-FCR-SSS-020 | Fire Control Radar Segment Specification | B |
| A3-BMC-SSS-030 | Battle Management Center Specification | A |
| A3-ENV-050 | Environmental and RAM Requirements | B |
| MIL-STD-1275E | Characteristics of 28 Volt DC Electrical Systems in Military Vehicles | E |
| MIL-STD-461G | Control of Electromagnetic Interference Characteristics | G |
| MIL-STD-810H | Environmental Engineering Considerations and Laboratory Tests | H |
| MIL-STD-882E | System Safety | E |
| NGA.STND.0036 | World Geodetic System 1984 (WGS-84) | 1.0.0 |

# 3. Requirements

Verification methods: A = Analysis, I = Inspection, D = Demonstration, T = Test.

## 3.1 Engagement timeline

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| SRS-TL-001 | The weapon system shall launch the first interceptor within 8.0 s of the engagement order issued by the BMC operator (system reaction time). | T | Mission need MN-04 |
| SRS-TL-002 | The system reaction time of SRS-TL-001 shall be allocated sequentially: BMC engagement planning (operator order to engagement command at the launcher) ≤ 2.0 s; launcher preparation (engagement command to PTL issued) ≤ 3.0 s; interceptor pre-launch preparation (PTL received to READY reported) ≤ 3.0 s. The launcher issues LCH on READY. | A | SRS-TL-001 |
| SRS-TL-003 | The system shall support simultaneous engagement of multiple threats; the number of simultaneous engagements is specified in A3-SRS-001-CA. | D | Mission need MN-05 |

## 3.2 Time and reference frames

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| SRS-TIM-001 | All segments shall maintain a common system time, disciplined to GPS by the BMC master clock, synchronised within ±1 µs of the master clock. | T | SRS-TL-001 |
| SRS-NAV-002 | All target state vectors and launch parameters exchanged between segments shall be expressed in WGS-84 Earth-Centred, Earth-Fixed (ECEF) coordinates, in metres and metres per second. | I | NGA.STND.0036 |

## 3.3 Electrical power

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| SRS-PWR-001 | Field equipment shall operate from 28 VDC power with the characteristics of MIL-STD-1275E. | T | — |
| SRS-PWR-002 | The launcher shall supply all interceptor pre-launch power through the umbilical defined in A3-ICD-110; the interceptor shall not draw more than the per-canister allocation of A3-ICD-110. | A | SRS-PWR-001 |

## 3.4 Interfaces

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| SRS-IF-001 | The interceptor–launcher interface shall conform to A3-ICD-110 Rev C. | I | — |
| SRS-IF-002 | Data interfaces between BMC, FCR and launchers shall conform to A3-ICD-120 Rev C. | I | — |
| SRS-IF-003 | All tactical messages shall be authenticated; encryption per the program cryptographic plan. | T | Security CONOPS |

## 3.5 Environment and EMC

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| SRS-ENV-001 | All field-deployed equipment shall meet the environmental requirements of A3-ENV-050 Rev B. | T | — |
| SRS-ENV-002 | All field-deployed equipment shall operate at ambient temperatures from −33 °C to +55 °C, with solar loading per MIL-STD-810H Method 505.7. | T | SRS-ENV-001 |
| SRS-EMC-001 | All segments shall meet MIL-STD-461G, tailored per A3-ENV-050 Rev B. | T | — |

## 3.6 Reliability, availability and maintainability

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| SRS-RAM-001 | The weapon system operational availability (Ao) shall be at least 0.95. | A | — |
| SRS-RAM-004 | Segment reliability allocations shall be as defined in A3-ENV-050 Rev B, Section 4. | A | SRS-RAM-001 |
| SRS-RAM-010 | The interceptor shall have a storage life of at least 15 years in its sealed canister, with periodic built-in test only. | A | Logistics concept |

## 3.7 Safety

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| SRS-SAF-001 | A system safety program shall be conducted per MIL-STD-882E. | I | — |
| SRS-SAF-002 | Rocket motor ignition shall require at least two independent inhibits to be removed, one of which is controlled by the launcher. | D | SRS-SAF-001 |

# 4. Notes

Open items: SRS-TL-003 values are TBR pending the threat assessment update (action SRR-112).

| Acronym | Meaning |
| --- | --- |
| AUR | All-up round (interceptor in its canister) |
| BMC | Battle Management Center |
| CCB | Configuration Control Board |
| ECEF | Earth-Centred, Earth-Fixed |
| FCR | Fire Control Radar |
| ICD | Interface Control Document |
| IFTU | In-flight target update |
| LCU | Launcher Control Unit |
| MTBF | Mean time between failures |
| PTL | Prepare-to-Launch command |
| RT | Remote terminal (MIL-STD-1553B) |
| TBR | To be reviewed |