ARROW 3 WEAPON SYSTEM

Interceptor Segment Specification

Two-stage interceptor and exo-atmospheric kill vehicle — draft for Preliminary Design Review

| Document number | A3-INT-SSS-010 |
| --- | --- |
| Revision | A |
| Date | 20 October 2026 |
| Status | Draft for PDR — submitted for consistency check |
| Prepared by | Interceptor Integrated Product Team |
| Classification | UNCLASSIFIED // FICTITIOUS DEMONSTRATION DATA |

Revision history

| Rev | Date | Description | CCB ref. |
| --- | --- | --- | --- |
| A | 20 Oct 2026 | First issue for Preliminary Design Review | — |

Approvals

| Role | Organisation | Signature | Date |
| --- | --- | --- | --- |
| Interceptor Segment lead | Interceptor IPT | e-signed | 20 Oct 2026 |
| Chief Systems Engineer | Weapon System Engineering | pending | — |

# 1. Scope

This specification defines the requirements of the Arrow 3 Interceptor Segment: the two-stage interceptor, the exo-atmospheric kill vehicle and the canister, delivered together as the all-up round (AUR).

Kinematic and lethality performance is specified in the classified annex A3-SRS-001-CA.

# 2. Applicable documents

| Document | Title | Rev |
| --- | --- | --- |
| A3-SRS-001 | System Requirements Specification | C |
| A3-ICD-110 | Interceptor–Launcher Interface Control Document | B |
| A3-ICD-120 | BMC–FCR–Launcher Data Interface Control Document | A |
| A3-ENV-050 | Environmental and RAM Requirements | B |
| MIL-STD-461F | Control of Electromagnetic Interference Characteristics | F |
| MIL-STD-1275E | 28 VDC Electrical Systems in Military Vehicles | E |

# 3. Requirements

Verification methods: A = Analysis, I = Inspection, D = Demonstration, T = Test.

## 3.1 General and physical

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| INT-GEN-001 | The interceptor shall be a two-stage, solid-propellant vehicle carrying an exo-atmospheric hit-to-kill kill vehicle. | I | A3-SRS-001 §1.1 |
| INT-PHY-002 | The mass of the all-up round, including the canister, shall not exceed 1,450 kg. | I | A3-ICD-110 |
| INT-PHY-003 | The canister envelope shall conform to drawing A3-LCH-DWG-2210 Rev D. | I | IF-MEC-001 |

## 3.2 Electrical

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| INT-PWR-010 | The interceptor shall operate from launcher-supplied 28 VDC power with the characteristics of MIL-STD-1275E. | T | SRS-PWR-001 |
| INT-PWR-012 | During pre-launch (IMU run-up and thermal conditioning) the interceptor shall draw not more than 18 A continuous from the launcher supply. | T | SRS-PWR-002 |
| INT-PWR-014 | Internal thermal batteries shall be activated on receipt of the LCH command and shall power the interceptor for the whole flight. | T | — |

## 3.3 Data and timing

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| INT-DAT-020 | The interceptor shall exchange pre-launch data with the launcher over a dual-redundant Gigabit Ethernet (1000BASE-T) link, the interceptor acting as server. | T | A3-ICD-110 |
| INT-DAT-022 | In-flight target update messages shall be decoded per A3-ICD-120 Rev A. | I | A3-ICD-120 |
| INT-TIM-031 | The interceptor shall report READY within 6.0 s of receiving the Prepare-to-Launch (PTL) command. | T | SRS-TL-002 |
| INT-TIM-035 | The interceptor time reference shall be synchronised to system time within ±10 µs. | T | SRS-TIM-001 |

## 3.4 Navigation and guidance

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| INT-NAV-040 | Pre-launch target data and in-flight target updates shall be received and processed in Earth-Centred Inertial coordinates (ECI, J2000 epoch). | I | SRS-NAV-002 |
| INT-NAV-042 | The inertial measurement unit shall be aligned before launch by transfer alignment from the launcher (message ALN). | T | IF-DAT-004 |

## 3.5 Environment and EMC

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| INT-ENV-050 | The all-up round shall operate at ambient temperatures from −20 °C to +50 °C. | T | SRS-ENV-002 |
| INT-ENV-052 | Non-operating (storage and transport) temperature range shall be −40 °C to +71 °C. | T | ENV-TMP-002 |
| INT-EMC-060 | The interceptor shall meet MIL-STD-461F requirements CE102, CS114, RE102 and RS103. | T | SRS-EMC-001 |

## 3.6 Reliability and safety

| Req. ID | Requirement | Verif. | Parent / trace |
| --- | --- | --- | --- |
| INT-RAM-070 | The captive-carry MTBF of the all-up round, powered on the launcher, shall be at least 1,500 h. | A | RAM-004 |
| INT-RAM-072 | The interceptor shall have a storage life of at least 10 years in its sealed canister. | A | SRS-RAM-010 |
| INT-SAF-080 | Rocket motor ignition shall require two independent inhibits to be removed, one controlled by the launcher. | D | SRS-SAF-002 |
| INT-CYB-090 | All messages received by the interceptor shall be authenticated before use. | T | SRS-IF-003 |

# 4. Notes

This draft is submitted for the PDR data package. Segment-level changes since the concept phase: Ethernet replaces the legacy data bus to support faster alignment transfer; the guidance team prefers inertial coordinates for in-flight updates.

| Acronym | Meaning |
| --- | --- |
| AUR | All-up round (interceptor in its canister) |
| BMC | Battle Management Center |
| CCB | Configuration Control Board |
| ECEF | Earth-Centred, Earth-Fixed |
| FCR | Fire Control Radar |
| ICD | Interface Control Document |
| IFTU | In-flight target update |
| LCU | Launcher Control Unit |
| MTBF | Mean time between failures |
| PTL | Prepare-to-Launch command |
| RT | Remote terminal (MIL-STD-1553B) |
| TBR | To be reviewed |